# FORCED VIBRATION

mx=fesinω t−kx−cx

k

c

mx+cx+kx=fesinω t

x+2ζωnx+ωn2x=fesinω tm

m

fe sint

The general solution of a nonhomogeneous equation consists

of two parts:

- the general solution of homogeneous equation

Xhexp(−ζωnt)sin(ωdt+ψ)   representing free vibration

- a particular solution of the nonhomogeneous equation

Assume 		xp(t)=Xsin(ωt−ϕ) as the particular solution

which represents the steady-state oscillation (free vibration part diminishes).

Solution		X=fe(k−mω2)2+(cω)2	       ϕ=arctan(cωk−mω2)

Solution of homogenous equation (free vibration)

Particular solution of nonhomogenous solution

Complete solution (forced vibration)   x(t)=xh(t)+xp(t)

ϕ=arctan[2ζωωn1−(ωωn)2]

kX/fe

cωk

/n nn



/n nn

These formulas indicate that the nondimensional amplitude kXfe and phase ϕ depend on the frequency ratio ωωn.

The curves can be divided into three regions:

(1) ωωn≺1 (smaller than and not close to 1) ; (2) ωωn≃1; and (3) ωωn≻1.

In the region of ωωn close to 1, the nondimensional amplitude becomes very large.

kXfe=12ζ	at ωωn=1

Damping has a great influence at resonance. If damping is absent, the amplitude becomes infinite at resonance in theory.

Where dose the maximum kXfe occur (ωωn)? What is the maximum?

Beyond ωωn=2, the nondimensional amplitude is smaller than one, that is, the amplitude is smaller than the static deflection.

The other thing to notice is that the response x(t) lags behind the excitation force by a phase angle ϕ<90∘ for ω<ωn and ϕ>90∘ for ω>ωn. At ω=ωn, there is a steep change of phase angle.

The natural frequency ωn (internal) and ω (external) should not be confused.

Machines with Rotating Parts

Suppose the center of mass is away from the

e

centre of rotation. The off-centred mass will

create an unbalanced force of



meω2cosωt in horizontal direction

meω2sinωt in vertical direction

(Complex) Frequency Response Function

(information only)

The excitation can be a sinusoidal or co-sinusoidal function of time. Both cases can be represented by a single function of time.

mx+cx+kx=feexp(iω t)  (i=−1)

The steady-state solution is assumed as x(t)=Xexp(iωt)

(k+icω−mω2)Xexp(iωt)=feexp(iω t)

X=fek+icω−mω2		x(t)=fek+icω−mω2exp(iωt)

The frequency response function is defined as

Xfe=1k+icω−mω2

Support Motion and Vibration Isolation

Sometimes a vibration is initiated by an oscillatory motion, from ground (earthquake) or an adjacent connection, rather than by an oscillatory force. Theses cases can be considered as support motion.

x

Equation of motion

m

mx=−k(x−y)−c(x−y)

k

c

let z=x−y

y

mz+cz+kz=−my

y(t)=Yexp(iωt)

Suppose z(t)=Zexp[i(ωt−ϕ)] and x(t)=Xexp[i(ωt−φ)]. Z and , and X and  can be found from the above equation.

XY=k2+(cω)2(k−mω2)2+(cω)2=1+[2ζ(ωωn)]2[1−(ωωn)2]2+[2ζ(ωωn)]2

tanφ=2ζ(ωωn)31−(ωωn)2+[2ζ(ωωn)]2

2

X/Y



ωωn

From the above graph of  XY, one can see for good isolation, k must be selected such that ωωn>2(then XY<1). Such a device is called a vibration isolator.

Transfer Function

When the excitation is a harmonic function of time, it is easy to find the response. For a general excitation of f(t), determining the response in the time-domain can be difficult.

The equation of motion of a mass-spring-damper system under a general excitation is

mx+cx+kx=f( t)

Taking the Laplace transform and assuming the initial response is zero, yields

X(s)=G(s)F(s)	G(s)=1ms2+cs+k

and

G(s)=X(s)F(s)	transfer function.

It can be noticed that G(iω) is the frequency response function defined in a previous section.

Once the excitationf(t)is given in a known system, F(s) can be found and X(s)can also be found from the system’s transfer function. By means of the inverse Laplace transform of X(s), the system’s response to f(t), x(t) can be determined.

Example:

Suppose the excitation to a mass-spring-damper system is shown below

a

t

f0

f(t)

1.   F(s)=0∞f(t)exp(−st)dt=f0a∞exp(−st)dt=f0exp(−sa)s

2.   G(s)=1ms2+cs+k

3.   X(s)=G(s)F(s)=f0exp(−sa)s(ms2+cs+k)

4.   x(t)=L−1[X(s)]=L-1[f0exp(−sa)s(ms2+cs+k)]=H(t−a)u(t−a)

where H is a unit step function and u is the response of a unit step function.